Lab 04

spice

# Contents

Contents	2

Part 1 (prelab)	5

1	Using ADT Device Xplore, plot the following design charts vs gm/ID for both PMOS and NMOS. Set VDS = VDD/3 and L = 0.18u,0.5u:0.5u:2u	5

1.1	Nmos	5

1.1.1	gm/gds	5

1.1.2	ID/W	6

1.1.3	gm/Cgg	6

1.1.4	VGS	7

1.2	PMOS	7

1.2.1	gm/gds	7

1.2.2	ID/W	8

1.2.3	gm/Cgg	8

1.2.4	VGS	9

2	Use ADT Device Xplore to find ID and VGS of an NMOS that has gm/ID = 10, gm/gds = 50, VDS = VDD/3, VSB = 0, and W = 5u.	9

3	Complete the script “lookup_test.m” to repeat the previous problem. Compare the results in a table.	11

4	Use ADT Device Xplore to design an NMOS-input 5T-OTA	12

4.1	gm/ID = 15 for the input pair and gm/ID = 10 for the load and the tail bias (why is this reasonable?).	12

4.2	ro of the load is five times ro of the input pair (why is this reasonable?).	12

4.3	L = 1um for the tail bias (why is this reasonable?).	12

4.4	DESIGN	13

4.5	using charts:	13

4.5.1	input pair:	13

4.5.2	Active Load:	15

4.5.3	Tail bias:	16

5	Complete the function “designOTA.m”, Report your MATLAB function.	17

6	Complete the script “designOTA_test.m”, Report your script. Report the output of the command window, Compare the results to the results obtained by ADT Device Xplore.	19

7	Complete the netlist “ota_tb.cir”, Simulate the differential gain vs frequency (annotate DC gain and GBW), Report your netlist, the simulation results, and the comparison table. Comment on the results.	21

PART2	25

1	Use ADT Device Xplore to redesign the previous OTA while taking into account the OTA self-loading , Compare the results to the results obtained in Part 1	25

1.1	COMPARISION WITH PART1:	30

2	MATLAB SCRIPT,. Compare the results to the results obtained by ADT Device Xplore	31

2.1	Report the output of the command window	31

2.2	Compare the results to the results obtained in Part 1.	31

2.3	Compare the results to the results obtained by ADT Device Xplore.	32

3	Write a netlist for the synthesized OTA. Simulate the differential gain vs frequency (annotate DC gain and GBW) . Comment on the results.	33

3.1	Report your netlists	33

3.2	the simulation results	34

3.3	comparison table	34

# Part 1 (prelab)

# Using ADT Device Xplore, plot the following design charts vs gm/ID for both PMOS and NMOS. Set VDS = VDD/3 and L = 0.18u,0.5u:0.5u:2u

## Nmos

### gm/gds

### ID/W

### gm/Cgg

### VGS

## PMOS

### gm/gds

### ID/W

### gm/Cgg

### VGS

# Use ADT Device Xplore to find ID and VGS of an NMOS that has gm/ID = 10, gm/gds = 50, VDS = VDD/3, VSB = 0, and W = 5u.

using magic cursor to get GM/GDS >50 we chose L=260n

At gm/id=10, id/w=19.15 and W=5u, so ID=95.75u

VGS=669.8m

# Complete the script “lookup_test.m” to repeat the previous problem. Compare the results in a table.

% Lookup Test

% Please copy this script to gm/ID starter directory & run it from there

% Clear workspace

clear all;

%load LUTs

load 180nch.mat;

load 180pch.mat;

% Compute VGS & ID

% Givens

VDD = 1.8;

M1.gm_ID = 10;

M1.gm_gds = 50;

M1.VDS = VDD / 3;

M1.VSB = 0;

M1.W = 5;

M1.gm_gds = 50;

% Find L that give gm/gds > given value

L_vector = nch.L; % Get the L vector from LUT structure

%Get the gm/gds values vector corresponding to the L_vector

gm_gds_vector = look_up(nch, 'GM_GDS', 'GM_ID', M1.gm_ID, 'VDS', M1.VDS, 'L', L_vector);

% Get the minimum L that gives gm/gds > the given value

M1.L=min(L_vector(gm_gds_vector>=M1.gm_gds)) ;

% Get the current by computing the ID/W and then multiply it by W

M1.ID_W = look_up(nch, 'ID_W', 'GM_ID', M1.gm_ID, 'VDS', M1.VDS, 'L', M1.L)

M1.ID=M1.ID_W*M1.W ;

% Get the VGS value

M1.VGS=look_upVGS(nch, 'GM_ID', M1.gm_ID, 'VDS', M1.VDS, 'L', M1.L)

% Print the solution

fprintf('VGS = %.2f\n',M1.VGS);

fprintf('ID = %.2d\n',M1.ID);

|  | MATLAB | ADT |
| --- | --- | --- |
| VGS (mv) | 670 | 670 |
| ID (uA) | 95.2u | 95.75u |

# Use ADT Device Xplore to design an NMOS-input 5T-OTA

## gm/ID = 15 for the input pair and gm/ID = 10 for the load and the tail bias (why is this reasonable?).

As for the input pair we need bigger gm for more gain and more GBW as, so we bias it in MI, for the tail we biased it into SI so we would have large output resistance so smaller AvCM, for the load we care more about the output resistance, so we bias it more towards the SI to get better λ for more gain. AS:

Adiff=-gmin* (rin //rload)  , GBW=gmin2πCL

## ro of the load is five times ro of the input pair (why is this reasonable?).

as the parallel combination would result in better Rout so better gain, though it doesn’t make sense to make rload much bigger than  rin as the input pair not biased in SI so it has small λin so small  ro and the result of the parallel is smaller than  ro.

## L = 1um for the tail bias (why is this reasonable?).

AvCM=-12gmloadRss

We need large L for the tail so we would have larger Rss so smaller AvCM, we would have better CMRR

## DESIGN

GBW=100Mhz=gmin2πCL , gmin=315 uS

(gmid)in=15 , ID=21 uA

AV=56ringmin=50.2 , rin=192 kΩ , rload=960kΩ

gmidload=10=gmload21u, gmload=210 uS

For the tail we assumed L=1 um, gmid=10 , and we know Itail=42 uA, so gmtail=420 uS

## using charts:

### input pair:

gmin=315 uS, rin=192 kΩ

gmgdsinmin=60.48

gmIDin=15

We choose L=280n, so gmgds=62.96,

IDW=7.03, so W=3.01 um

### Active Load:

gmgdsloadmin=201.6, gmid=10

L=1.7u

IDW=669.3m, so W=31.38 um

The load trans has large size, but we didn’t consider the parasitics until know in this part.

### Tail bias:

L=1u,

ID=42uA,

gmID=10

IDW=4.665, so W=9 um

# Complete the function “designOTA.m”, Report your MATLAB function.

%OTA Synthesis Function

function OTA = designOTA(specs)

%Additional Specs

VDD = 1.8;

%load LUTs

load 180nch.mat;

load 180pch.mat;

%Input Pair

OTA.M1.gm = specs.GBW * specs.CL * 2 * pi;

% assume ro(load) = 5 * ro(input) --> ro(input) = 6/5 R_total

DC_Gain_mag = 10^(specs.AVDC / 20); % Convert from dB to mag

Rout = DC_Gain_mag / OTA.M1.gm; % Compute the equivalent output resistance of OTA (Rout)

OTA.M1.ro =(6/5)*Rout % Complete the line to compute the ro of M1

OTA.M1.gds = 1 / OTA.M1.ro;

OTA.M1.VDS = VDD/3;

OTA.M1.gm_gds = OTA.M1.gm / OTA.M1.gds;

OTA.M1.gm_ID = 15; % assumption

OTA.M1.ID =OTA.M1.gm /OTA.M1.gm_ID % Complete the line to get the current of M1

% Search for the minimum L that gives gm / gds > specified value

L_vector = nch.L;

gm_gds_vector = look_up(nch, 'GM_GDS', 'GM_ID', OTA.M1.gm_ID, 'VDS', OTA.M1.VDS, 'L', L_vector);

OTA.M1.L = min(L_vector(gm_gds_vector>=OTA.M1.gm_gds)) ;

% Compute ID/W to get the W value

OTA.M1.ID_W = look_up(nch, 'ID_W', 'GM_ID', OTA.M1.gm_ID, 'VDS', OTA.M1.VDS, 'L', OTA.M1.L);

OTA.M1.W = OTA.M1.ID / OTA.M1.ID_W;

%CM_Load

OTA.M3.ID = OTA.M1.ID;

OTA.M3.ro = 5*OTA.M1.ro

OTA.M3.gds = 1 / OTA.M3.ro;

OTA.M3.VDS = VDD/3;

OTA.M3.gm_ID = 10;

OTA.M3.gm = OTA.M3.gm_ID * OTA.M3.ID;

OTA.M3.gm_gds = OTA.M3.gm / OTA.M3.gds;

gm_gds_vector = look_up(pch, 'GM_GDS', 'GM_ID', OTA.M3.gm_ID, 'VDS', OTA.M3.VDS, 'L', L_vector);

OTA.M3.L = min(L_vector(gm_gds_vector > OTA.M3.gm_gds));

OTA.M3.ID_W = look_up(pch, 'ID_W', 'GM_ID', OTA.M3.gm_ID, 'VDS', OTA.M3.VDS, 'L', OTA.M3.L);

OTA.M3.W = OTA.M3.ID / OTA.M3.ID_W;

% Tail bias

OTA.M5.L = 1; %assumption

OTA.M5.ID = 2 * OTA.M1.ID;

OTA.M5.VDS = VDD/3;

OTA.M5.gm_ID = 10; %assumption

% Get ID/W to compute W

OTA.M5.ID_W = look_up(nch, 'ID_W', 'GM_ID', OTA.M5.gm_ID, 'VDS', OTA.M5.VDS, 'L', OTA.M5.L);

OTA.M5.W = OTA.M5.ID / OTA.M5.ID_W;

% get CMIN bias value

OTA.M1.VGS =look_upVGS(nch, 'GM_ID', OTA.M1.gm_ID, 'VDS', OTA.M1.VDS, 'L', OTA.M1.L)

OTA.M1.VG = OTA.M1.VGS + OTA.M5.VDS

# Complete the script “designOTA_test.m”, Report your script. Report the output of the command window, Compare the results to the results obtained by ADT Device Xplore.

% OTA Design Script

% Write the SPECS

clear all;

AVDC = 34 ;

GBW = 100e6 ;

CL = 500e-15 ;

specs = struct('AVDC', AVDC,...

'CL', CL,...

'GBW', GBW);

OTA = designOTA(specs);

% Print the solution

fprintf('**** OTA Design ****\n\n');

fprintf('Input Pair:\n');

fprintf('    L = %.2f um\n    W=%.2f um\n    ViCM=%.4f V\n\n',OTA.M1.L,OTA.M1.W,OTA.M1.VG);

fprintf('CM Load:\n');

fprintf('    L = %.2f um\n    W=%.2f um\n\n',OTA.M3.L,OTA.M3.W);

fprintf('Tail Current Source:\n');

fprintf('    L = %.2f um\n    W=%.2f um\n\n',OTA.M5.L,OTA.M5.W);

|  |  | ADT | SCRIPT |
| --- | --- | --- | --- |
| M1(INPUT) | L (um) | 0.28 | 0.28 |
|  | W (um) | 3.01 | 3.01 |
| M3(LOAD) | L (um) | 1.7 | 1.7 |
|  | W (um) | 31.38 | 31.44 |
| M5(TAIL) | L (um) | 1 | 1 |
|  | W (um) | 9 | 9.06 |

The results are almost the same, just second-degree differences because of small approximations.

# Complete the netlist “ota_tb.cir”, Simulate the differential gain vs frequency (annotate DC gain and GBW), Report your netlist, the simulation results, and the comparison table. Comment on the results.

|  | SPICE | SPECS |
| --- | --- | --- |
| GBW | 1.818*51.64=93.88 MHZ | 100 MHZ |
| GAIN | 34.26 DB | 34 DB |

COMMENT:

GBW is less than 100 MHZ because of the self-parasitic capacitances that we didn’t consider.

CDBin=2.3fF

CDBin=1.45fF

CDBload=25.74fF

CGDload=20.8fF

CLEFF=CL+CDBin+CGDin+CDBload+CGDload=550.29fF

GBW=gm2πCLeff= 91MHZ, I didn’t consider body effect.

CLEFFCL-1=10.058%

# PART2

# USE ADT DEVICE XPLORE TO REDESIGN THE PREVIOUS OTA WHILE TAKING INTO ACCOUNT THE OTA SELF-LOADING , COMPARE THE RESULTS TO THE RESULTS OBTAINED IN PART 1

## first iteration:

GBW=100MHZ=gmin2πCLeff,gmin=346uS

ID=23.07 uA

56gmgds=50.2 ,sogmgds is the same so L is the same
LIN=280 nm, WIN= 3.285 um

CDBin+CGDin=2.562+1.577=4.139 fF

gmgdsin=62.96 , gdsin=5.495 uS,gdsload=1.1 uS

gmidload=10, gmload=230.7 uS,gmgdsload=209.72

So we choose L=1.8u,

W=36.55 uS, for Capacitances calculations:

CDBLOAD+CGDLOAD=30+24.2=54.2

CLeff=558.34fF ,CLCLeff-1=1.516%

Second iteration:

GBW=100MHZ=gmin2πCLeff,gmin=350.8uS

ID=23.39 uA

gdsin=5.572 uS,gdsload=1.114, gmload=233.9uS

gmgdsload=209.96 uS, same L and same charts

WIN=3.33 ,WLOAD=37.06

CLeff=500+24.494+30.37+2.597+1.6=559.061 fF

CLCLeff-1=0.13%

Third iteration:

gmin=351.26uS

ID=23.418 uA

same L and same charts

WIN=3.335 ,WLOAD=37.1

CLeff=500+24.56+30.46+2.6+1.6=559.22 fF

CLCLeff-1=0.028%

WTAIL=23.418*24.665=10.04 uS

## COMPARISION WITH PART1:

|  |  | PART1 | PART2 |
| --- | --- | --- | --- |
| M1(INPUT) | L (um) | 0.28 | 0.28 |
|  | W (um) | 3.01 | 3.335 |
| M3(LOAD) | L (um) | 1.7 | 1.8 |
|  | W (um) | 31.38 | 37.1 |
| M5(TAIL) | L (um) | 1 | 1 |
|  | W (um) | 9 | 10.04 |

# MATLAB SCRIPT,. Compare the results to the results obtained by ADT Device Xplore

## Report the output of the command window

## Compare the results to the results obtained in Part 1.

|  |  | PART1 | PART2 |
| --- | --- | --- | --- |
| M1(INPUT) | L (um) | 0.28 | 0.28 |
|  | W (um) | 3.01 | 3.12 |
| M3(LOAD) | L (um) | 1.7 | 1.7 |
|  | W (um) | 31.38 | 34.89 |
| M5(TAIL) | L (um) | 1 | 1 |
|  | W (um) | 9 | 10.06 |

## Compare the results to the results obtained by ADT Device Xplore.

|  |  | Script | ADT |
| --- | --- | --- | --- |
| M1(INPUT) | L (um) | 0.28 | 0.28 |
|  | W (um) | 3.12 | 3.335 |
| M3(LOAD) | L (um) | 1.7 | 1.8 |
|  | W (um) | 34.89 | 37.1 |
| M5(TAIL) | L (um) | 1 | 1 |
|  | W (um) | 10.06 | 10.04 |

MATLAB code is more optimized due it didn’t take any margin or approximation that I took.

# Write a netlist for the synthesized OTA. Simulate the differential gain vs frequency (annotate DC gain and GBW) . Comment on the results.

## Report your netlists

## the simulation results

Gain =34.26 DB, GBW=1.98MHZ * 51.64=102.25MHZ

## comparison table

|  | SPICE | SPECS |
| --- | --- | --- |
| GBW | 102.25 MHZ | 100 MHZ |
| GAIN | 34.26 DB | 34 DB |

Results meet the specs, using lookup tables was very helpful and practical for design.

Another approach by changing the bias point

# Use ADT Device Xplore to redesign the previous OTA while taking into account the OTA self-loading , Compare the results to the results obtained in Part 1

To get less current, so PMOS transistor would have less size, we need to  bias the input transistor more into WI, so we choose:   gmid=16, as CLeff=550.29fF, GBW=gmin2πCLeff, gmin=345.75 uS

we choose gmin=350 uS, so Id=21.875 uA

gmgds=63.6, gdsin=5.5 uS

W=3.8u

We have slightly bigger W but anyway nmos caps weren’t the main factor of GBW decreasing.

gmidload=10, gm=218.75uS

gdsload=1.1 uS ,gmgdsload=198.86

So we choose L=1.7u to get less W

IDW=669.3m, Wload=32.7 um

For tail we have I=43.75 mA

W=9.37 um

Calculate load capacitance again:

CGDLOAD=21.63 fF

CDBLOAD=26.8fF

CDBIN=2.96fF

CGDIN=1.82fF

CLeff=553.21fF, gm=350 S

GBW=gm2πCLeff=100.7MHZ

## COMPARISION WITH PART1:

|  |  | PART1 | PART2 |
| --- | --- | --- | --- |
| M1(INPUT) | L (um) | 0.28 | 0.28 |
|  | W (um) | 3.01 | 3.8 |
| M3(LOAD) | L (um) | 1.7 | 1.7 |
|  | W (um) | 31.38 | 32.7 |
| M5(TAIL) | L (um) | 1 | 1 |
|  | W (um) | 9 | 9.37 |

# MATLAB SCRIPT,. Compare the results to the results obtained by ADT Device Xplore

## Report the output of the command window

## Compare the results to the results obtained in Part 1.

|  |  | PART1 | PART2 |
| --- | --- | --- | --- |
| M1(INPUT) | L (um) | 0.28 | 0.28 |
|  | W (um) | 3.01 | 3.8 |
| M3(LOAD) | L (um) | 1.7 | 1.5 |
|  | W (um) | 31.38 | 28.49 |
| M5(TAIL) | L (um) | 1 | 1 |
|  | W (um) | 9 | 9.36 |

## Compare the results to the results obtained by ADT Device Xplore.

|  |  | Script | ADT |
| --- | --- | --- | --- |
| M1(INPUT) | L (um) | 0.28 | 0.28 |
|  | W (um) | 3.8 | 3.8 |
| M3(LOAD) | L (um) | 1.5 | 1.7 |
|  | W (um) | 28.49 | 32.7 |
| M5(TAIL) | L (um) | 1 | 1 |
|  | W (um) | 9.36 | 9.37 |

MATLAB code is more optimized due it didn’t take any margin or approximation that I took.

# Write a netlist for the synthesized OTA. Simulate the differential gain vs frequency (annotate DC gain and GBW) . Comment on the results.

## Report your netlists

## the simulation results

GAIN=34.42 DB

GBW=52.63*1.98MHZ=104.2MHZ

## comparison table

|  | SPICE | SPECS |
| --- | --- | --- |
| GBW | 104.2 MHZ | 100 MHZ |
| GAIN | 34.42 DB | 34 DB |

Results meet the specs, using lookup tables was very helpful and practical for design.